ENGR 4399 ST: Cyber-Physical and IoT Systems
Project 1 Report

| Course | ENGR 4399 ST: Cyber-Physical and IoT Systems |
| --- | --- |
| Instructor | Dr. Okan Caglayan |
| Semester/Term | Fall 2026 |
| Assignment # / Title | Project 1 — Automatic Plant Watering System with Weather-Aware Dashboard (Smart Greenhouse, Stage 1) |
| Due Date | 10/20/2026 |
| Student Name(s) | de Oliveira, Luis; Garcia, Uriel |
| Group Members (if applicable) | Luis de Oliveira; Uriel Garcia |
| Wokwi Project URL | Final system built on real ESP32 hardware. Wokwi prototype of the base state machine: [TO FILL – Wokwi project URL] |
| Microcontroller Board/Model | ESP32 development board (30-pin, USB-C) |
| Components Used | Keyestudio resistive soil moisture sensor; Keyestudio water level sensor; Elegoo 1-channel 5 V relay module (active-HIGH); small DC submersible pump with tube; 0.96" I2C OLED (SSD1306, 0x3C); active buzzer; Elegoo MB102 breadboard power module and 9 V battery (optional); breadboard, jumper wires, cup of water, plant/soil |

Abstract

This project builds an automatic plant watering system on a real ESP32 as stage 1 of a Smart Greenhouse. A resistive soil moisture sensor and a water level sensor feed a four-state finite state machine (IDLE, WATERING, SOAKING, FAULT) that switches a submersible pump through a relay, using hysteresis, an 8 s pump run limit, a soak delay, a tank-level interlock and a latching fault. The ESP32 also reads a live weather forecast over Wi-Fi to skip watering when rain is likely, judges the temperature against one of three per-plant profiles and drives a buzzer alert, and serves a web dashboard with simulation buttons (rain, tank low, soil too dry, temperature too high or too low) so every protective behavior can be demonstrated on demand. The concept is inspired by the Instagram video “Automatic Plant Watering System” by eazytronic3795. Verification combines bench measurements of the sensors and relay with a test plan of expected-versus-observed cases. The work exercises ADC sensing and calibration, digital output and relay isolation, I2C, a finite state machine, non-blocking timing, and Wi-Fi/HTTP/JSON networking.

Design Description and Analysis

1. System Overview

Houseplants fail from too little water when nobody is around and from too much water when the soil is already wet. A useful automatic waterer must therefore do more than switch a pump when the soil is dry: it must avoid running the pump dry, avoid flooding the plant, notice when the hardware is broken, and take context such as incoming rain and unhealthy temperatures into account. These are classic cyber-physical concerns: sensing, decision, actuation and safe failure.

The basic idea (soil moisture sensor, relay and small pump watering a potted plant) is inspired by the Instagram video “Automatic Plant Watering System” by eazytronic3795. We rebuilt it on an ESP32 and added a tank-level interlock, hysteresis with run limit and soak delay, a response check with a fault latch, an OLED status screen, a weather-forecast rain block, temperature judgement with a buzzer alert, three plant profiles and a web dashboard with simulation buttons. The system takes two analog inputs (soil moisture and tank level) plus a weather forecast, decides with a state machine and acts on a relay-driven pump, an OLED, a buzzer and the dashboard. Figure 1 shows the block diagram. This is stage 1 of the Smart Greenhouse; a grow light and richer plant profiles are planned for stage 2.

Figure 1. System block diagram showing sensors, ESP32 controller, relay and pump, user interfaces (OLED, buzzer, dashboard) and the weather service.

2. Circuit Description

Both Keyestudio sensors are 3-pin analog modules (S = signal, V = power, G = ground) powered from 3V3. Their signal pins go to D34 and D35, which are ADC1 channels: they are input-only and, unlike ADC2, remain usable while Wi-Fi is active. The relay module is powered from VIN (5 V) and its signal input is driven from D26; it was measured to be ACTIVE-HIGH (the relay closes when D26 is HIGH). The pump has its own supply routed through the relay contacts (supply + to COM, NO to the pump red wire, pump black wire to supply −), so the pump current never flows through the ESP32. NO (normally open) is used so the pump is off whenever the relay is de-energized. The OLED uses the default I2C pins (SDA D21, SCL D22) at address 0x3C. The active buzzer is on D27 and the on-board BOOT button (D0, INPUT_PULLUP) is reused as a manual 5 s pump test. All ESP32-side grounds are common. The 9 V battery with the MB102 power module is optional and, if used, supplies only the pump circuit.

Table 1. Pin Function Mapping

| Arduino Pin | Component | Function/Signal | Notes |
| --- | --- | --- | --- |
| D34 (ADC1) | Soil moisture sensor S | Analog input | V to 3V3, G to GND. About 0 in air, about 1500 in water (reads LOW when dry) |
| D35 (ADC1) | Water level sensor S | Analog input | V to 3V3, G to GND. Tank LOW below raw 300, refilled above 450 |
| D26 | Relay header S | Digital output | ACTIVE-HIGH. Relay header + to VIN (5 V), − to GND |
| Relay screws | COM / NO | Pump switch contacts | Pump supply (+) to COM; NO to pump red wire; pump black wire to supply (−) |
| D21 (SDA) | OLED SSD1306 | I2C data | Address 0x3C; VCC to 3V3, GND to GND |
| D22 (SCL) | OLED SSD1306 | I2C clock | Shared bus with SDA |
| D27 | Active buzzer | Digital output | Two short beeps = too hot, one long beep = too cold; other lead to GND |
| D0 (BOOT) | On-board button | Digital input (INPUT_PULLUP) | Manual pump test, max 5 s, blocked when tank is low |
| 3V3 / VIN / GND | Power rails | Power | 3V3: sensors and OLED. VIN (5 V): relay module. Common GND |
| USB (UART0) | Programming / Serial Monitor | Serial 115200 baud | Power and upload; event and status log |

Figure 2. Wiring schematic of the real hardware showing every connection and pin mapping (net labels show power and ground connections).

[INSERT PHOTO – Figure 3: assembled hardware (ESP32, sensors, relay, pump, tank, plant)]

Figure 3. Assembled hardware on the breadboard. [TO FILL: replace with your photo]

Design note. The relay polarity and the soil sensor direction were verified on the bench before the pump was connected (see Section 7). The soil probe is resistive and its exposed prongs corrode over time; this is acknowledged as a limitation.

3. Code Description

The firmware (Appendix A) is an Arduino C++ sketch for the “ESP32 Dev Module” board. It uses the libraries Wire.h, WiFi.h, WebServer.h, ESPmDNS.h and HTTPClient.h (ESP32 core) and Adafruit_GFX / Adafruit_SSD1306 for the OLED. Program structure:

setup(). Switches the relay off first (the pump is guaranteed off at boot), configures the buzzer pin and the BOOT button, starts Serial at 115200 baud, sets 12-bit ADC resolution, starts I2C and the OLED, then calls startNetwork(). startNetwork() tries to join the configured Wi-Fi for up to 12 s; on failure it opens its own access point “PlantWater”. It registers plantwater.local with mDNS and starts the web server.

loop(). Non-blocking: it services the web server and the BOOT button on every pass, runs sampleAndControl() every 500 ms (millis()-based), fetches weather when due and the pump is idle, updates the buzzer, and finally switches the relay in exactly one place from a single expression that combines the state machine, the tank interlock, the rain block and the manual test.

sampleAndControl(). Reads the soil sensor (16-sample average) and the level sensor (8-sample average), converts the soil raw value to percent with the calibration constants SOIL_RAW_DRY = 0 and SOIL_RAW_WET = 1500, applies hysteresis to the tank status (LOW below 300, OK above 450), and runs the state machine in Figure 4.

Helpers. relayWrite() maps “on/off” to the pin level for an active-HIGH or active-LOW relay; readAveraged() low-pass filters the ADC; effectiveTemp(), effectiveRain() and rainBlock() return the simulated value if a dashboard simulation is active, otherwise the real one; tempStatus() classifies the temperature as unknown, OK, too high or too low for the selected profile; fetchWeather() requests the Open-Meteo forecast and parses the current temperature and the maximum rain probability of the next 3 hours; setState() logs every transition to the Serial Monitor and to an 8-entry event log shown on the dashboard.

Figure 4. Pump finite state machine with transition conditions (IDLE, WATERING, SOAKING, FAULT).

Control parameters.

Start / stop moisture per profile (baseline 30 % / 60 %); hysteresis prevents the pump chattering around one threshold.

Pump run limit 8 s; soak 30 s (demo value, minutes for a real plant); after a soak the soil must have risen at least 3 percentage points or the cycle counts as failed; three failed cycles latch FAULT, cleared only by the EN reset.

Rain block: a rain probability of 60 % or more in the next 3 hours blocks automatic watering and stops a running pump. Weather is refreshed every 10 min (30 s after a failure) and never while the pump runs.

Plant profiles (start watering below / stop at / healthy temperature): Basil 40 % / 70 % / 12–32 °C; Aloe 15 % / 35 % / 10–35 °C; Fern 50 % / 80 % / 16–27 °C. These defaults are tunable in the PROFILES table. [TO FILL: change names or values if different plants are used]

Temperature alert. Outside the profile range the dashboard and OLED show the alert and the buzzer sounds with a pattern derived from millis() % 2000: two short beeps for too hot, one long beep for too cold. A dashboard button silences it until the status changes.

4. Simulation Setup

The final system runs on real hardware; a Wokwi prototype of the base state machine (potentiometer in place of the soil sensor, LED in place of the pump) was used first. [TO FILL: Wokwi URL]. To run the real system:

Wire the circuit as in Table 1 and Figure 2 with the pump disconnected. In the Arduino IDE select “ESP32 Dev Module”, install the Adafruit SSD1306 and Adafruit GFX libraries, set WIFI_SSID and WIFI_PASS (a phone hotspot or router), upload RealHardware_Dashboard.ino and open the Serial Monitor at 115200 baud.

Calibrate: read raw values with the soil probe in air and in water, and with the level sensor at the lowest acceptable water level; store them in SOIL_RAW_DRY, SOIL_RAW_WET and LEVEL_LOW_RAW.

Verify the relay polarity with the pump disconnected, then connect the pump (supply + to COM, NO to pump red) and hold BOOT to prime the tube.

Open http://plantwater.local (or the IP printed on the OLED and Serial Monitor) from a device on the same Wi-Fi. If the Wi-Fi is unavailable, join the “PlantWater” network and open http://192.168.4.1; weather is then unavailable and only the simulations apply.

Inputs and observation. Real inputs are the soil moisture and tank level. The dashboard provides five simulation buttons that inject a condition on top of the real readings: Rain forecast (rain probability 100 %), Tank low, Soil too dry (soil reads 10 points below the start threshold), Temp too high and Temp too low (8 °C beyond the profile limit). Outputs are observed on the OLED, the dashboard, the buzzer, the relay click and the pump, and in the Serial Monitor. The simulations cannot disable the pump run limit, the tank interlock or the fault latch.

Figure 5. Dashboard layout (UI preview rendered in a desktop browser): normal operation (left) and rain-forecast simulation (right). [TO FILL: replace with real screenshots from the ESP32]

5. Results & Verification

Verification combines bench measurements on the real board with the test plan in Table 2. Test cases 1 and 2 were performed during the build (relay polarity and soil sensor calibration). The remaining expected behaviors are derived from the firmware logic; their observed results and Pass/Fail entries are completed from the real tests and are marked below. No result is reported that was not observed. [TO FILL]

Table 2. Test Cases and Outcomes

| Test Case # | Steps / Inputs | Expected Behavior | Observed Behavior | Pass/Fail |
| --- | --- | --- | --- | --- |
| 1 | Relay-only test sketch toggles D26 HIGH/LOW, pump disconnected | Relay clicks ON with D26 HIGH; OFF at boot | Relay is ACTIVE-HIGH (first assumed active-low); OFF at boot | Pass |
| 2 | Soil probe in air, then in a glass of water; read raw ADC | Distinct dry and wet values | About 0 in air, about 1500 in water (reads LOW when dry) | Pass |
| 3 | Level sensor dry, then at the minimum safe water level | Dry about 0; reading rises with depth; LOW below 300 | [TO FILL: raw readings] | [TO FILL] |
| 4 | Hold BOOT with water in the tank | Pump runs while held, stops after 5 s | [TO FILL] | [TO FILL] |
| 5 | Full cycle: dry soil, pump connected, tank OK, no rain | IDLE to WATERING (pump on), SOAKING, IDLE; stops at the stop % or 8 s | [TO FILL: serial log, soil % before/after] | [TO FILL] |
| 6 | Real tank low: lift the level sensor out of the water | OLED TANK LOW; pump blocked; no fault counted | [TO FILL] | [TO FILL] |
| 7 | FAULT: dry soil, tube pinched or pump out of water | After 3 failed cycles state FAULT; pump off until EN reset | [TO FILL] | [TO FILL] |
| 8 | Dashboard: Rain forecast | Watering blocked; running pump stops; event logged | [TO FILL] | [TO FILL] |
| 9 | Dashboard: Tank low | Pump blocked; no fault counted | [TO FILL] | [TO FILL] |
| 10 | Dashboard: Soil too dry (tank OK, no rain) | Relay clicks, pump starts and stops at the 8 s limit; no false fault | [TO FILL] | [TO FILL] |
| 11 | Dashboard: Temp too high | TOO HIGH on dashboard and OLED; two short beeps; Silence button mutes | [TO FILL] | [TO FILL] |
| 12 | Dashboard: Temp too low | TOO LOW alert; one long beep | [TO FILL] | [TO FILL] |
| 13 | Switch profile Basil, Aloe, Fern | Thresholds, markers and healthy range change | [TO FILL] | [TO FILL] |
| 14 | Live weather over Wi-Fi | Temperature and rain % match a reference weather source | [TO FILL: values and source compared] | [TO FILL] |
| 15 | Wrong Wi-Fi password at boot | Own network “PlantWater” starts; dashboard at 192.168.4.1 | [TO FILL] | [TO FILL] |

[INSERT SERIAL MONITOR EXCERPT – calibration readings, one watering cycle, TANK LOW and FAULT events]

Figure 6. Serial Monitor output showing sensor readings, state transitions and events. [TO FILL]

[INSERT PHOTOS – OLED in each state (normal, WATERING, TANK LOW, RAIN, TEMP alert, FAULT)]

Figure 7. OLED status screens. [TO FILL]

[INSERT PHOTO / VIDEO STILL – pump starting when the soil is dry]

Figure 8. Pump running during a watering cycle. [TO FILL]

6. Discussion of Engineering Concepts

The design exercises the following course concepts, each tied to the code:

ADC resolution, quantization and calibration. analogReadResolution(12) gives 4096 levels (about 0.8 mV per step at 3.3 V). The raw value is converted by soilPercentFromRaw() using two measured points (0 in air, 1500 in water) rather than a datasheet formula, because the ESP32 ADC is not perfectly linear and this probe reads the opposite direction to YL-69 style sensors. constrain() clips the result to 0–100 %. ADC1 pins are used because ADC2 cannot be used while Wi-Fi is on.

Filtering. readAveraged() averages 16 (soil) or 8 (level) samples, a simple low-pass filter that suppresses noise and spikes.

Finite state machine. sampleAndControl() implements IDLE, WATERING, SOAKING and FAULT with guarded transitions (Figure 4). Evaluating safety conditions (tank low, rain) before success and time-limit conditions makes the priority explicit. FAULT is a latched state cleared only by hardware reset.

Hysteresis. Separate start and stop thresholds (for Basil 40 % and 70 %) and the tank status thresholds (300 and 450) stop the outputs chattering around a single threshold.

Digital output and relay isolation. D26 drives the relay input; the relay contacts carry the pump current from a separate supply, isolating the ESP32. relayWrite() abstracts active-HIGH and active-LOW polarity, and the relay is driven from one place in loop().

Pull-up input. The BOOT button on D0 uses INPUT_PULLUP, so it idles HIGH and reads LOW when pressed; updatePrime() bounds the manual run to 5 s.

I2C. The OLED shares the two-wire SDA/SCL bus at address 0x3C using the Adafruit SSD1306 driver; the screen shows soil %, state, tank status and the highest-priority alert.

Wi-Fi, HTTP, REST and JSON. The ESP32 is an HTTP client of the Open-Meteo forecast API (fetchWeather() reads the current temperature and the maximum rain probability by locating keys in the JSON text) and an HTTP server for the dashboard, with a small JSON API (/api/status, /api/sim, /api/profile, /api/mute, /api/weather). mDNS provides the name plantwater.local.

Real-time behavior and timing. All timing uses millis(); loop() never blocks, so the web server, buzzer pattern and safety checks stay responsive. The blocking weather request is skipped while the pump runs so the network never delays the pump limits. The buzzer pattern is derived from millis() % 2000.

Safety and failure handling. Relay forced off at boot, tank interlock, rain block, 8 s run limit, response check and fault latch form layered mitigations (Table 3).

Security. The dashboard is unauthenticated HTTP and the weather request is plain HTTP, acceptable for a classroom demonstration but not for deployment (see Section 7).

7. Troubleshooting & Limitations

Issues encountered and how they were diagnosed.

The relay turned out to be ACTIVE-HIGH, not active-low as first assumed. A relay-only test sketch run with the pump disconnected showed the relay closes when D26 is HIGH; RELAY_ACTIVE_LOW is set to false.

This soil sensor reads LOW when dry (about 0 in air) and HIGH when wet (about 1500 in water), the opposite of YL-69 style sensors used in many tutorials. The calibration constants and the percentage mapping use the measured values.

The ESP32 must be on a 2.4 GHz network and the dashboard host must be on the same network; when the router was not reachable the board fell back to its own “PlantWater” network, and plantwater.local proved more reliable than a changing IP address.

[TO FILL: add any wiring or software problem encountered when connecting the pump, e.g. brown-out, tube placement, simulated soil behavior]

Failure modes and mitigations.

Table 3. Failure Modes, Detection and Mitigation

| Failure mode | Cause | Detection | Mitigation |
| --- | --- | --- | --- |
| Pump runs dry | Empty tank | Level sensor below 300 | Pump blocked and stopped; not counted as a fault; resumes after refill |
| Pump never stops | Sensor stuck, probe out of soil | Run timer | 8 s maximum run per cycle |
| Pump or tube failure | Dead pump, kinked tube, probe far from drip | Soil rise under 3 % after a soak | CHECK PUMP message; FAULT after 3 cycles until EN reset |
| Overwatering | Single-threshold chatter; watering before rain | Hysteresis, soak, forecast | Start/stop thresholds, soak delay, rain block |
| Wrong temperature | Heat wave or cold night | Temperature vs profile range | Dashboard verdict, OLED alert, buzzer |
| Wi-Fi or weather down | Network loss, API error | Fetch failure | Watering continues on soil logic; weather unknown; own-network fallback |
| Relay polarity or boot glitch | Wrong polarity assumption | Bench relay test first | Polarity constant; relay off first in setup() |
| Brown-out at pump start | Pump on weak 5 V source | Unexpected reset | Separate pump supply; relay isolates the circuits |
| Spoofed data | Unauthenticated dashboard | Not detected in this stage | See security note |

Limitations.

The temperature judged is the forecast outdoor temperature for the location, not a sensor at the plant; a DHT22-type sensor at the plant is the natural stage 2 improvement.

The rain block uses forecast probability, not measured rain at the pot, and is meaningful mainly for outdoor plants.

Plant profile values are defaults and need tuning with real plants; the 30 s soak is a demo value.

The resistive soil probe corrodes over time; a capacitive probe is preferable for long-term use.

The dashboard has no login and uses plain HTTP, and the weather request is not encrypted. A real product would add authentication, HTTPS, removal of the simulation interface and signed firmware updates (secure boot, flash encryption, signed OTA). The simulations cannot disable the run limit, tank interlock or fault latch.

Test coverage with real watering cycles is limited by tank size and time. [TO FILL: note any tests not completed]

Conclusion

The system extends the basic soil-sensor-and-pump idea into a small cyber-physical system with sensing, forecast awareness, per-plant decisions, actuation and layered safety. The sensors and the relay were verified on the real board, and the firmware implements a tank interlock, run limit, response check and latching fault, with a dashboard whose simulations make each protective behavior demonstrable on demand. [TO FILL: one sentence stating whether all tests passed.] The most important takeaway is that verifying assumptions on the real hardware (relay polarity, sensor direction) mattered as much as the code. The next improvement is a temperature and humidity sensor at the plant plus a grow light, leading into stage 2 of the Smart Greenhouse, together with authentication and HTTPS for the dashboard.

Appendix A — Arduino Code Listing

Complete RealHardware_Dashboard.ino (Wi-Fi credentials replaced with placeholders):

/*

* ENGR 4399 - Project 1: Automatic Plant Watering System - REAL ESP32 + WEB DASHBOARD

* (stage 1 of the Smart Greenhouse)

*

* Inspired by the Instagram video "Automatic Plant Watering System" by eazytronic3795.

*

* Added on top of the tested RealHardware.ino:

*   - Wi-Fi weather forecast (Open-Meteo, no API key): rain chance blocks automatic watering

*   - temperature judgement per plant profile + active buzzer alert

*   - 3 per-plant profiles (start/stop moisture, healthy temperature range)

*   - web dashboard served by the ESP32 itself, with SIMULATION buttons:

*       rain forecast, tank low, soil too dry, temperature too high / too low

*   - if the Wi-Fi does not connect the ESP32 opens its own network "PlantWater"

*

* Wiring (same as the tested build, plus the buzzer)

*   Soil sensor     S -> D34   V -> 3V3   G -> GND

*   Level sensor    S -> D35   V -> 3V3   G -> GND

*   Relay header    S -> D26   +  -> VIN (5 V)   - -> GND   (ACTIVE-HIGH module)

*   Relay screws    COM -> pump supply (+), NO -> pump red wire, pump black -> supply (-)

*   OLED SSD1306    SDA -> D21  SCL -> D22  VCC -> 3V3  GND -> GND  (0x3C)

*   Active buzzer   signal / (+) -> D27   (-) / G -> GND

*

* Board: "ESP32 Dev Module". Libraries: "Adafruit SSD1306", "Adafruit GFX Library".

* WiFi, WebServer, ESPmDNS and HTTPClient come with the ESP32 core.

*/

#include <Arduino.h>

#include <Wire.h>

#include <WiFi.h>

#include <WebServer.h>

#include <ESPmDNS.h>

#include <HTTPClient.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// ============================ Options (edit these) =========================

static const char* WIFI_SSID = "YOUR_WIFI_NAME";   // network the ESP32 joins ("" = own network only)

static const char* WIFI_PASS = "YOUR_WIFI_PASSWORD";

static const char* AP_SSID   = "PlantWater";   // own network if the Wi-Fi fails

static const char* AP_PASS   = "plant1234";    // at least 8 characters

static const uint32_t WIFI_TIMEOUT_MS = 12000;

// Weather location (default: University of the Incarnate Word, San Antonio TX)

static const char* LATITUDE  = "29.47";

static const char* LONGITUDE = "-98.47";

static const int   RAIN_SKIP_PCT = 60;         // rain chance (next 3 h) at/above this blocks automatic watering

static const uint8_t OLED_ADDR = 0x3C;

static const bool RELAY_ACTIVE_LOW     = false; // measured: this relay is ACTIVE-HIGH

static const bool RELAY_FLOAT_WHEN_OFF = false;

static const bool BUZZER_ACTIVE_LOW    = false;

// Soil calibration (this Keyestudio probe reads LOW when dry, HIGH when wet)

static const int SOIL_RAW_DRY = 0;

static const int SOIL_RAW_WET = 1500;

// Water-level sensor

static const int LEVEL_LOW_RAW  = 300;

static const int LEVEL_HYST_RAW = 150;

// Control

static const uint32_t MAX_RUN_MS        = 8000;

static const uint32_t SOAK_MS           = 30000;

static const float    MIN_RISE_PCT      = 3.0f;

static const int      MAX_FAILED_CYCLES = 3;

static const uint32_t PRIME_MAX_MS      = 5000;

static const uint32_t SAMPLE_MS         = 500;

static const uint32_t LOG_MS            = 2000;

// ============================== Plant profiles =============================

struct Profile { const char* name; const char* need; float dry; float wet; float tMin; float tMax; };

static const Profile PROFILES[3] = {

// name      water need           start  stop  min C  max C

{ "Basil",  "thirsty",            40.0f, 70.0f, 12.0f, 32.0f },

{ "Aloe",   "drought-tolerant",   15.0f, 35.0f, 10.0f, 35.0f },

{ "Fern",   "likes moist soil",   50.0f, 80.0f, 16.0f, 27.0f },

};

static uint8_t profileIdx = 0;

// ================================== Pins ===================================

static const int PIN_SOIL   = 34;

static const int PIN_LEVEL  = 35;

static const int PIN_RELAY  = 26;

static const int PIN_BUZZER = 27;

static const int PIN_PRIME  = 0;     // BOOT button

static const int PIN_SDA    = 21;

static const int PIN_SCL    = 22;

static Adafruit_SSD1306 oled(128, 64, &Wire, -1);

static bool oledOk = false;

static WebServer server(80);

// ================================== State ==================================

enum State : uint8_t { IDLE, WATERING, SOAKING, FAULT };

static const char* STATE_NAME[] = {"IDLE", "WATERING", "SOAKING", "FAULT"};

static State    state = IDLE;

static uint32_t stateSince = 0;

static float    soilAtStart = 0;

static bool     evalPending = false;

static bool     noResponse = false;

static int      failedCycles = 0;

static int      cycles = 0;

static bool     primeOn = false;

static bool     realTankLow = false;

static bool     tankLow = false;          // real OR simulated

static float    soilReal = 0, soilShown = 0;

static int      soilRaw = 0, levelRaw = 0;

// Simulation switches (set from the dashboard)

static bool    simRain = false, simTank = false, simSoil = false;

static uint8_t simTemp = 0;               // 0 off, 1 too high, 2 too low

// Weather

static float    weatherTemp = NAN;

static int      weatherRain = -1;         // max rain probability next 3 h, -1 unknown

static uint32_t weatherOkAt = 0, nextWeatherAt = 0;

static bool     weatherEver = false;

static bool     apMode = false;

static String   ipText = "-";

// Alerts

static uint8_t lastTempStatus = 0;

static bool    muted = false;

static bool    buzzing = false;

// Event log shown on the dashboard

static char    events[8][80];

static uint8_t evHead = 0, evCount = 0;

static void logEvent(const char* msg) {

strncpy(events[evHead], msg, 79); events[evHead][79] = 0;

evHead = (evHead + 1) % 8; if (evCount < 8) evCount++;

Serial.printf("[EVENT] %s\n", msg);

}

// ================================ Helpers ==================================

static void relayWrite(bool on) {

if (RELAY_FLOAT_WHEN_OFF && RELAY_ACTIVE_LOW) {

pinMode(PIN_RELAY, OUTPUT_OPEN_DRAIN);

digitalWrite(PIN_RELAY, on ? LOW : HIGH);

return;

}

pinMode(PIN_RELAY, OUTPUT);

bool level = RELAY_ACTIVE_LOW ? !on : on;

digitalWrite(PIN_RELAY, level ? HIGH : LOW);

}

static void buzzerWrite(bool on) {

digitalWrite(PIN_BUZZER, (on != BUZZER_ACTIVE_LOW) ? HIGH : LOW);

}

static int readAveraged(int adcPin, int samples) {

long acc = 0;

for (int i = 0; i < samples; i++) { acc += analogRead(adcPin); delayMicroseconds(200); }

return (int)(acc / samples);

}

static float soilPercentFromRaw(int raw) {

float pct = 100.0f * (raw - SOIL_RAW_DRY) / (float)(SOIL_RAW_WET - SOIL_RAW_DRY);

return constrain(pct, 0.0f, 100.0f);

}

static float curDry()  { return PROFILES[profileIdx].dry; }

static float curWet()  { return PROFILES[profileIdx].wet; }

static float curTMin() { return PROFILES[profileIdx].tMin; }

static float curTMax() { return PROFILES[profileIdx].tMax; }

static float effectiveTemp() {

if (simTemp == 1) return curTMax() + 8.0f;

if (simTemp == 2) return curTMin() - 8.0f;

return weatherTemp;

}

static int effectiveRain() { return simRain ? 100 : weatherRain; }

static bool rainBlock()    { int r = effectiveRain(); return r >= RAIN_SKIP_PCT; }

// 0 unknown, 1 OK, 2 too high, 3 too low

static uint8_t tempStatus() {

float t = effectiveTemp();

if (isnan(t)) return 0;

if (t > curTMax()) return 2;

if (t < curTMin()) return 3;

return 1;

}

static void setState(uint8_t s, const char* why) {

char b[80];

snprintf(b, sizeof b, "%s -> %s: %s", STATE_NAME[state], STATE_NAME[s], why);

logEvent(b);

state = (State)s;

stateSince = millis();

}

// ================================ Weather ==================================

static void fetchWeather() {

if (WiFi.status() != WL_CONNECTED) return;

String url = String("http://api.open-meteo.com/v1/forecast?latitude=") + LATITUDE + "&longitude=" + LONGITUDE +

"&current=temperature_2m&hourly=precipitation_probability&forecast_hours=3&timezone=auto";

HTTPClient http;

http.setTimeout(4000);

http.begin(url);

int code = http.GET();

bool ok = false;

if (code == 200) {

String body = http.getString();

int c = body.indexOf("\"current\":{");

if (c >= 0) {

int t = body.indexOf("\"temperature_2m\":", c);

if (t >= 0) { weatherTemp = body.substring(t + 17).toFloat(); ok = true; }

}

int h = body.indexOf("\"hourly\":{");

if (h >= 0) {

int p = body.indexOf("\"precipitation_probability\":[", h);

if (p >= 0) {

int i = p + 29, best = -1;

while (i < (int)body.length() && body[i] != ']') {

if (isDigit(body[i])) { int v = body.substring(i).toInt(); if (v > best) best = v; while (isDigit(body[i])) i++; }

else i++;

}

if (best >= 0) weatherRain = best;

}

}

}

http.end();

if (ok) {

weatherEver = true; weatherOkAt = millis(); nextWeatherAt = millis() + 600000UL;

Serial.printf("[WEATHER] %.1f C, rain chance next 3 h: %d %%\n", weatherTemp, weatherRain);

logEvent("Weather updated");

} else {

nextWeatherAt = millis() + 30000UL;

Serial.printf("[WEATHER] fetch failed (HTTP %d)\n", code);

}

}

// ================================ Dashboard ================================

static const char PAGE[] PROGMEM = R"HTML(<!doctype html><html lang="en"><head><meta charset="utf-8">

<meta name="viewport" content="width=device-width,initial-scale=1"><title>Plant Watering Dashboard</title>

<style>

:root{--bg:#f4f6f3;--card:#fff;--ink:#16221a;--mut:#5c6b60;--line:#dfe5df;--ok:#1f7a45;--warn:#b26a00;--bad:#b3261e;--acc:#1f7a45;--sim:#6b3fa0}

@media(prefers-color-scheme:dark){:root{--bg:#101612;--card:#18211b;--ink:#e8efe9;--mut:#9bab9f;--line:#2a372d;--ok:#4cc27f;--warn:#e0a13a;--bad:#ff7b72;--acc:#4cc27f;--sim:#b58cf0}}

*{box-sizing:border-box}body{margin:0;background:var(--bg);color:var(--ink);font:15px/1.4 system-ui,sans-serif}

header{padding:14px 16px;display:flex;justify-content:space-between;align-items:center;gap:8px;flex-wrap:wrap}

h1{font-size:18px;margin:0}small,.mut{color:var(--mut)}

main{display:grid;gap:12px;padding:0 16px 24px;max-width:960px;margin:auto;grid-template-columns:repeat(auto-fit,minmax(280px,1fr))}

.card{background:var(--card);border:1px solid var(--line);border-radius:10px;padding:14px}

.card h2{font-size:13px;letter-spacing:.04em;text-transform:uppercase;color:var(--mut);margin:0 0 8px}

.big{font-size:34px;font-weight:700}.row{display:flex;justify-content:space-between;gap:8px;padding:3px 0}

.badge{display:inline-block;padding:2px 9px;border-radius:99px;font-weight:600;font-size:13px;border:1px solid currentColor}

.ok{color:var(--ok)}.warn{color:var(--warn)}.bad{color:var(--bad)}.simc{color:var(--sim)}

.bar{position:relative;height:16px;border:1px solid var(--line);border-radius:4px;margin:8px 0 14px;background:var(--bg)}

.bar i{display:block;height:100%;background:var(--acc);border-radius:3px}.bar b{position:absolute;top:-4px;width:2px;height:22px;background:var(--ink)}

button{font:inherit;border:1px solid var(--line);background:var(--card);color:var(--ink);padding:9px 12px;border-radius:8px;cursor:pointer;margin:3px 3px 3px 0;min-height:40px}

button.on{background:var(--sim);color:#fff;border-color:var(--sim)}button.sel{background:var(--acc);color:#fff;border-color:var(--acc)}

.log div{font:12px ui-monospace,monospace;padding:2px 0;border-bottom:1px dashed var(--line)}

.wide{grid-column:1/-1}#temp{font-size:26px}

</style></head><body>

<header><h1>Automatic Plant Watering &middot; Smart Greenhouse stage 1</h1><small id="conn">connecting...</small></header>

<main>

<section class="card"><h2>Weather &amp; temperature</h2>

<div class="big" id="temp">--</div><div id="tverdict"></div>

<div class="row"><span class="mut">Healthy range</span><span id="trange">--</span></div>

<div class="row"><span class="mut">Rain chance (3 h)</span><span id="rain">--</span></div>

<div class="row"><span class="mut">Watering</span><span id="rainverdict">--</span></div>

</section>

<section class="card"><h2>Soil &amp; pump</h2>

<div class="big" id="soil">--</div>

<div class="bar"><i id="soilbar" style="width:0"></i><b id="m1"></b><b id="m2"></b></div>

<div class="row"><span class="mut">State</span><span id="state">--</span></div>

<div class="row"><span class="mut">Pump</span><span id="pump">--</span></div>

<div class="row"><span class="mut">Tank</span><span id="tank">--</span></div>

<div class="row"><span class="mut">Failed cycles</span><span id="fail">--</span></div>

</section>

<section class="card"><h2>Plant profile</h2><div id="profiles"></div><div class="mut" id="pinfo"></div></section>

<section class="card"><h2>Simulations (inject a condition)</h2>

<button id="s_rain" onclick="sim('rain',1)">Rain forecast</button>

<button id="s_tank" onclick="sim('tank',1)">Tank low</button>

<button id="s_soil" onclick="sim('soil',1)">Soil too dry</button>

<button id="s_th" onclick="sim('temp',1)">Temp too high</button>

<button id="s_tl" onclick="sim('temp',2)">Temp too low</button><br>

<button onclick="api('/api/sim?name=clear')">Clear all</button>

<button onclick="api('/api/mute')" id="mute">Silence buzzer</button>

<button onclick="api('/api/weather')">Refresh weather</button>

<div class="mut" id="simnote"></div>

</section>

<section class="card wide log"><h2>Event log</h2><div id="log"></div></section>

</main>

<script>

const $=i=>document.getElementById(i);let S={};

async function api(p){try{await fetch(p)}catch(e){}refresh()}

function sim(n,v){const cur=n==='temp'?S.sim.temp:(S.sim[n]?1:0);api('/api/sim?name='+n+'&val='+(cur===v?0:v))}

function F(c){return (c*9/5+32).toFixed(1)}

async function refresh(){try{S=await (await fetch('/api/status')).json();draw(S);$('conn').textContent=S.net+' · '+S.ip}catch(e){$('conn').textContent='offline'}}

function draw(s){const p=s.profiles[s.profile];

$('temp').textContent=s.temp===null?'--':s.temp.toFixed(1)+' °C ('+F(s.temp)+' °F)'+(s.sim.temp?' *':'');

const tv=['<span class="badge mut">no data</span>','<span class="badge ok">Temperature OK</span>','<span class="badge bad">TOO HIGH for '+p.name+'</span>','<span class="badge bad">TOO LOW for '+p.name+'</span>'];

$('tverdict').innerHTML=tv[s.tstatus];

$('trange').textContent=p.tmin+' - '+p.tmax+' °C';

$('rain').innerHTML=s.rain<0?'unknown':s.rain+' %'+(s.sim.rain?' <span class="simc">(simulated)</span>':'');

$('rainverdict').innerHTML=s.rainblock?'<span class="badge warn">BLOCKED - rain will water it</span>':'<span class="badge ok">allowed</span>';

$('soil').textContent=s.soil.toFixed(0)+' %'+(s.sim.soil?' *':'');

$('soilbar').style.width=s.soil+'%';$('m1').style.left=p.dry+'%';$('m2').style.left=p.wet+'%';

$('state').innerHTML='<b class="'+(s.state==='FAULT'?'bad':'')+'">'+s.state+'</b>';

$('pump').innerHTML=s.pump?'<span class="badge ok">ON</span>':'off';

$('tank').innerHTML=s.tank?'<span class="badge bad">LOW'+(s.sim.tank?' (sim)':'')+'</span>':'<span class="badge ok">OK</span>';

$('fail').textContent=s.failed+' / 3';

$('profiles').innerHTML=s.profiles.map((q,i)=>'<button class="'+(i==s.profile?'sel':'')+'" onclick="api(\'/api/profile?id='+i+'\')">'+q.name+'</button>').join('');

$('pinfo').textContent=p.need+': water below '+p.dry+' %, stop at '+p.wet+' %, healthy '+p.tmin+'-'+p.tmax+' °C';

$('s_rain').className=s.sim.rain?'on':'';$('s_tank').className=s.sim.tank?'on':'';$('s_soil').className=s.sim.soil?'on':'';

$('s_th').className=s.sim.temp==1?'on':'';$('s_tl').className=s.sim.temp==2?'on':'';

$('mute').textContent=s.buzzing?'Silence buzzer':(s.muted?'Buzzer silenced':'Silence buzzer');

$('simnote').textContent=(s.sim.rain||s.sim.tank||s.sim.soil||s.sim.temp)?'* Simulated values are active (purple buttons).':'No simulation active - real sensors in use.';

$('log').innerHTML=s.events.map(e=>'<div>'+e+'</div>').join('')}

refresh();setInterval(refresh,1000);

</script></body></html>)HTML";

static void sendStatus() {

float t = effectiveTemp();

String j; j.reserve(1800);

j += "{\"soil\":" + String(soilShown, 1);

j += ",\"state\":\"" + String(STATE_NAME[state]) + "\"";

j += ",\"pump\":" + String((state == WATERING && !tankLow && !rainBlock()) || primeOn ? "true" : "false");

j += ",\"tank\":" + String(tankLow ? "true" : "false");

j += ",\"rain\":" + String(effectiveRain());

j += ",\"rainblock\":" + String(rainBlock() ? "true" : "false");

j += ",\"temp\":" + (isnan(t) ? String("null") : String(t, 1));

j += ",\"tstatus\":" + String(tempStatus());

j += ",\"profile\":" + String(profileIdx);

j += ",\"failed\":" + String(failedCycles);

j += ",\"muted\":" + String(muted ? "true" : "false");

j += ",\"buzzing\":" + String(buzzing ? "true" : "false");

j += ",\"net\":\"" + String(apMode ? "own network PlantWater" : "Wi-Fi") + "\",\"ip\":\"" + ipText + "\"";

j += ",\"sim\":{\"rain\":" + String(simRain ? "true" : "false") + ",\"tank\":" + String(simTank ? "true" : "false") +

",\"soil\":" + String(simSoil ? "true" : "false") + ",\"temp\":" + String(simTemp) + "}";

j += ",\"profiles\":[";

for (int i = 0; i < 3; i++) {

if (i) j += ",";

j += "{\"name\":\"" + String(PROFILES[i].name) + "\",\"need\":\"" + PROFILES[i].need + "\",\"dry\":" + String(PROFILES[i].dry, 0) +

",\"wet\":" + String(PROFILES[i].wet, 0) + ",\"tmin\":" + String(PROFILES[i].tMin, 0) + ",\"tmax\":" + String(PROFILES[i].tMax, 0) + "}";

}

j += "],\"events\":[";

for (int k = 0; k < evCount; k++) {                 // newest first

int idx = (evHead - 1 - k + 16) % 8;

if (k) j += ",";

j += "\""; j += events[idx]; j += "\"";

}

j += "]}";

server.send(200, "application/json", j);

}

static void handleSim() {

String n = server.arg("name");

int v = server.arg("val").toInt();

if (n == "clear") { simRain = simTank = simSoil = false; simTemp = 0; logEvent("SIM: all simulations cleared"); }

else if (n == "rain") { simRain = v; logEvent(v ? "SIM: heavy rain forecast injected" : "SIM: rain forecast cleared"); }

else if (n == "tank") { simTank = v; logEvent(v ? "SIM: tank LOW injected" : "SIM: tank low cleared"); }

else if (n == "soil") { simSoil = v; logEvent(v ? "SIM: soil too dry injected" : "SIM: soil dry cleared"); }

else if (n == "temp") { simTemp = (v == 1 || v == 2) ? v : 0;

logEvent(simTemp == 1 ? "SIM: temperature TOO HIGH injected" : simTemp == 2 ? "SIM: temperature TOO LOW injected" : "SIM: temperature cleared"); }

server.send(200, "text/plain", "ok");

}

static void startNetwork() {

WiFi.mode(WIFI_STA);

if (strlen(WIFI_SSID) > 0) {

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.printf("Connecting to Wi-Fi \"%s\"", WIFI_SSID);

uint32_t t0 = millis();

while (WiFi.status() != WL_CONNECTED && millis() - t0 < WIFI_TIMEOUT_MS) { delay(300); Serial.print("."); }

Serial.println();

}

if (WiFi.status() == WL_CONNECTED) {

ipText = WiFi.localIP().toString();

apMode = false;

} else {

WiFi.disconnect(); WiFi.mode(WIFI_AP);

WiFi.softAP(AP_SSID, AP_PASS);

ipText = WiFi.softAPIP().toString();

apMode = true;

Serial.println("Wi-Fi unavailable: own network started (weather will be simulated only)");

}

if (MDNS.begin("plantwater")) Serial.println("Also reachable at http://plantwater.local");

server.on("/", []() { server.send_P(200, "text/html", PAGE); });

server.on("/api/status", sendStatus);

server.on("/api/sim", handleSim);

server.on("/api/profile", []() {

int id = server.arg("id").toInt();

if (id >= 0 && id < 3) { profileIdx = id; char b[80]; snprintf(b, sizeof b, "Profile changed to %s", PROFILES[id].name); logEvent(b); }

server.send(200, "text/plain", "ok");

});

server.on("/api/mute", []() { muted = true; logEvent("Buzzer silenced from dashboard"); server.send(200, "text/plain", "ok"); });

server.on("/api/weather", []() { nextWeatherAt = 0; server.send(200, "text/plain", "ok"); });

server.begin();

Serial.printf("Dashboard: http://%s\n", ipText.c_str());

}

// ============================ Display & buttons ============================

static void updatePrime(uint32_t now) {

static uint32_t start = 0;

static bool held = false;

if (digitalRead(PIN_PRIME) == LOW) {

if (!held) {

held = true; start = now;

Serial.println(tankLow ? "[PRIME] refused: tank is low" : "[PRIME] manual pump (max 5 s per press)");

}

primeOn = !tankLow && (now - start < PRIME_MAX_MS);

} else { held = false; primeOn = false; }

}

static void drawScreen(uint32_t now) {

if (!oledOk) return;

oled.clearDisplay();

oled.setTextColor(SSD1306_WHITE);

oled.setTextSize(1);

oled.setCursor(0, 0);  oled.print(PROFILES[profileIdx].name);

float t = effectiveTemp();

oled.setCursor(84, 0);

if (isnan(t)) oled.print("--C"); else oled.printf("%.0fC", t);

oled.setTextSize(2);

oled.setCursor(0, 11); oled.printf("Soil %.0f%%", soilShown);

oled.drawRect(0, 30, 128, 9, SSD1306_WHITE);

oled.fillRect(1, 31, (int)(126.0f * soilShown / 100.0f), 7, SSD1306_WHITE);

oled.drawFastVLine((int)(1 + 126.0f * curDry() / 100.0f), 40, 3, SSD1306_WHITE);

oled.drawFastVLine((int)(1 + 126.0f * curWet() / 100.0f), 40, 3, SSD1306_WHITE);

oled.setTextSize(1);

oled.setCursor(0, 45);

bool pumpOn = (state == WATERING && !tankLow && !rainBlock()) || primeOn;

oled.printf("%s  Pump:%s", STATE_NAME[state], pumpOn ? "ON" : "off");

oled.setCursor(0, 56);

uint8_t ts = tempStatus();

if (state == FAULT)        oled.print("FAULT - press EN");

else if (tankLow)          oled.print("TANK LOW - REFILL");

else if (rainBlock())      oled.print("RAIN - WATERING OFF");

else if (ts == 2)          oled.print("TEMP TOO HIGH!");

else if (ts == 3)          oled.print("TEMP TOO LOW!");

else if (noResponse)       oled.print("CHECK PUMP / TUBE");

else if (state == SOAKING) oled.printf("Tank OK  soak %lus", (unsigned long)((SOAK_MS - (now - stateSince)) / 1000));

else                       oled.print("Tank OK");

oled.display();

}

// ============================ Sampling & control ===========================

static void sampleAndControl(uint32_t now) {

static uint32_t lastLog = 0;

static bool lastTankLow = false, lastRainBlock = false;

soilRaw  = readAveraged(PIN_SOIL, 16);

levelRaw = readAveraged(PIN_LEVEL, 8);

soilReal = soilPercentFromRaw(soilRaw);

soilShown = simSoil ? max(0.0f, curDry() - 10.0f) : soilReal;     // "soil too dry" simulation

if (levelRaw < LEVEL_LOW_RAW) realTankLow = true;

else if (levelRaw > LEVEL_LOW_RAW + LEVEL_HYST_RAW) realTankLow = false;

tankLow = realTankLow || simTank;

if (tankLow != lastTankLow) {

logEvent(tankLow ? "TANK LOW - pump blocked, refill" : "Tank OK again");

lastTankLow = tankLow;

}

bool rb = rainBlock();

if (rb != lastRainBlock) {

logEvent(rb ? "Rain expected - automatic watering blocked" : "No rain expected - watering allowed");

lastRainBlock = rb;

}

switch (state) {

case IDLE:

if (soilShown < curDry() && !tankLow && !rb) { soilAtStart = soilShown; setState(WATERING, "soil dry"); }

break;

case WATERING:

if (tankLow) {

evalPending = false; setState(IDLE, "tank low - pump stopped");

} else if (rb) {

evalPending = false; setState(IDLE, "rain expected - pump stopped");

} else if (soilShown >= curWet()) {

failedCycles = 0; noResponse = false; evalPending = false; cycles++;

setState(SOAKING, "target moisture reached");

} else if (now - stateSince >= MAX_RUN_MS) {

evalPending = !simSoil;                // a simulated reading cannot prove or disprove the pump

cycles++;

setState(SOAKING, "max run time reached");

}

break;

case SOAKING:

if (now - stateSince >= SOAK_MS) {

if (evalPending) {

evalPending = false;

if (soilShown - soilAtStart < MIN_RISE_PCT) {

failedCycles++; noResponse = true;

char b[80]; snprintf(b, sizeof b, "Soil did not respond (%d/%d): check pump, tube, probe", failedCycles, MAX_FAILED_CYCLES);

logEvent(b);

} else { failedCycles = 0; noResponse = false; }

}

if (failedCycles >= MAX_FAILED_CYCLES) setState(FAULT, "too many failed cycles - press EN after fixing");

else setState(IDLE, "soak finished");

}

break;

case FAULT:

break;

}

drawScreen(now);

if (now - lastLog >= LOG_MS) {

lastLog = now;

Serial.printf("[SOIL] %.0f%% (raw %d) tank=%s (raw %d) state=%s pump=%s rain=%d%% temp=%.1f profile=%s sim[r%d t%d s%d T%d]\n",

soilShown, soilRaw, tankLow ? "LOW" : "ok", levelRaw, STATE_NAME[state],

((state == WATERING && !tankLow && !rb) || primeOn) ? "ON" : "off",

effectiveRain(), effectiveTemp(), PROFILES[profileIdx].name, simRain, simTank, simSoil, simTemp);

}

}

static void updateAlert(uint32_t now) {

uint8_t ts = tempStatus();

if (ts != lastTempStatus) {

muted = false;

if (ts == 2) logEvent("ALERT: temperature too high for this plant");

else if (ts == 3) logEvent("ALERT: temperature too low for this plant");

else if (lastTempStatus >= 2) logEvent("Temperature back in the healthy range");

lastTempStatus = ts;

}

bool want = false;

if ((ts == 2 || ts == 3) && !muted) {

uint32_t ph = now % 2000;

if (ts == 2) want = (ph < 150) || (ph >= 300 && ph < 450);   // two short beeps = too hot

else         want = (ph < 700);                              // one long beep  = too cold

}

buzzing = (ts == 2 || ts == 3) && !muted;

buzzerWrite(want);

}

void setup() {

relayWrite(false);                           // pump OFF first

pinMode(PIN_BUZZER, OUTPUT); buzzerWrite(false);

pinMode(PIN_PRIME, INPUT_PULLUP);

Serial.begin(115200);

analogReadResolution(12);

Wire.begin(PIN_SDA, PIN_SCL);

oledOk = oled.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR);

if (!oledOk) Serial.println("OLED not found - continuing without it");

Serial.println("\n=== Automatic Plant Watering + Dashboard - ENGR 4399 Project 1 ===");

if (oledOk) {

oled.clearDisplay(); oled.setTextColor(SSD1306_WHITE); oled.setTextSize(1);

oled.setCursor(0, 0); oled.print("Starting Wi-Fi..."); oled.display();

}

startNetwork();

logEvent("System started");

if (oledOk) {

oled.clearDisplay(); oled.setCursor(0, 0); oled.print("Dashboard:");

oled.setCursor(0, 14); oled.print(ipText); oled.setCursor(0, 30);

oled.print(apMode ? "Wi-Fi: PlantWater" : "plantwater.local"); oled.display();

delay(2500);

}

nextWeatherAt = millis() + 1000;

}

void loop() {

static uint32_t lastSample = 0;

static int lastRelay = -1;

uint32_t now = millis();

server.handleClient();

updatePrime(now);

if (now - lastSample >= SAMPLE_MS) { lastSample = now; sampleAndControl(now); }

// Weather is fetched only while the pump is off (the fetch can block for a few seconds)

bool pumpBusy = (state == WATERING) || primeOn;

if (!pumpBusy && !apMode && (int32_t)(now - nextWeatherAt) >= 0) fetchWeather();

updateAlert(now);

int wantOn = ((state == WATERING && !tankLow && !rainBlock()) || primeOn) ? 1 : 0;

if (wantOn != lastRelay) { relayWrite(wantOn); lastRelay = wantOn; }

}

Appendix B — References

eazytronic3795, “Automatic Plant Watering System,” Instagram video (inspiration for the project concept). [TO FILL: paste the video link]

Espressif Systems, “ESP32 Series Datasheet” and ESP-IDF Programming Guide, Release v5.3.2 (2024); Arduino-ESP32 core documentation (WiFi, WebServer, HTTPClient, ESPmDNS), https://docs.espressif.com/projects/arduino-esp32/

Open-Meteo, Free Weather Forecast API documentation, https://open-meteo.com/en/docs (weather data licensed CC BY 4.0).

Adafruit, Adafruit SSD1306 and Adafruit GFX libraries, https://github.com/adafruit

Keyestudio, KS0487 37-in-1 Sensor Kit v3.0 documentation (soil moisture and water level sensors); Elegoo, ESP32 development board and relay module documentation, https://wiki.elegoo.com/en/oshw-getting-started-&-kits

E. White, Making Embedded Systems, 2nd ed., O’Reilly, 2024; W. Dunn, Introduction to Instrumentation, Sensors, and Process Control, Artech House, 2005.

Wokwi simulator documentation, https://docs.wokwi.com ; Fritzing, https://fritzing.org/

Anthropic — Claude (Cowork), used to assist in developing the firmware, compiling and formatting this report, and generating the figures. All engineering content was reviewed by the authors.